Table of Contents

# EasyBluesky: A Graphical Interface for Bluesky-based Beamline Control at Synchrotron Facilities

Mrinal Bera [Your institution — e.g., Advanced Photon Source, Argonne National Laboratory / University of Chicago]

## Abstract

The Bluesky ecosystem provides a powerful and flexible framework for data acquisition at synchrotron beamlines, but its command-line and Python-scripting interface presents a significant barrier to researchers without programming experience. We present EasyBluesky, an open-source, cross-platform desktop application that exposes the full capability of the Bluesky queue-server through an intuitive graphical user interface. EasyBluesky manages the complete lifecycle of the remote RunEngine Manager — from SSH-based start-up and environment control to experiment logging and live data visualization — without requiring the user to interact with a terminal. A simulation mode allows plan development and testing against synthetic ophyd devices before committing to beamtime. For shared beamlines, a multi-client safety layer provides operator locking, real-time client visibility, and status disambiguation when multiple workstations share the same queue-server. Experiment Safety Assessment Form (ESAF) integration automates folder creation and injects proposal metadata into every plan. An integrated Visual Plan Composer assembles measurement sequences from graphical building blocks without requiring the user to write Python. An interactive curve-fitting module with per-dataset parameter memory supports rapid analysis within the same interface used for data collection. EasyBluesky is implemented in Python using PyQt6 and communicates with the queue-server via ZMQ. It is freely available at https://github.com/nayanbera/easy-bluesky under the BSD licence.

Keywords: beamline control, data acquisition, Bluesky, graphical user interface, Python, queue-server, synchrotron, EPICS

## 1. Introduction

Modern synchrotron beamlines generate data at rates and volumes that demand automated, reproducible data acquisition workflows. The Bluesky ecosystem (Allan et al., 2019) has emerged as a community standard for this purpose, offering a run engine that orchestrates hardware through ophyd device abstractions (Brookhaven National Laboratory, 2014) and records provenance-rich experimental metadata. The bluesky-queueserver extension (Contribute et al., 2021) further decouples the RunEngine from the user interface, exposing it over a ZMQ network interface that permits remote control from any machine on the network.

Despite this technical maturity, the day-to-day operation of a Bluesky-controlled beamline still demands a working knowledge of Python, IPython, and the command-line tools used to start, configure, and monitor the RunEngine. This requirement effectively restricts self-directed operation to researchers with software experience — a constraint that is increasingly at odds with the broad scientific user base of synchrotron facilities, which includes chemists, biologists, and materials scientists who are domain experts but not programmers.

Several graphical tools for Bluesky exist, including the Bluesky Widgets library and PyDM-based displays used at individual beamlines, but these are typically tightly coupled to specific facility configurations and require significant local customisation before deployment. A general-purpose, self-contained application that can be installed by an end user and pointed at any Bluesky queue-server over SSH has not previously been available.

Here we describe EasyBluesky (version 0.1.2), a PyQt6 desktop application that provides a complete graphical interface to a Bluesky queue-server running on a remote beamline computer. EasyBluesky targets the common scenario in which an experimenter works at a local workstation — or carries their own laptop — while the RunEngine executes on dedicated beamline hardware. The application has been developed and deployed at the ASWAXS beamline [your beamline details] and is made available as open-source software for adoption at other facilities.

## 2. Software Description

### 2.1 Architecture

EasyBluesky follows a client–server architecture in which all hardware interaction occurs on the remote beamline computer and the graphical application runs locally on the experimenter’s workstation (Fig. 1). The design separates concerns across three communication channels: ZMQ sockets for queue-server control and status polling, SSH for process lifecycle management and log streaming, and EPICS Channel Access (CA) for direct, low-latency device readback.

Figure 1. EasyBluesky client–server architecture. Three independent channels link the workstation to the beamline computer: ZMQ (queue control and live run documents), SSH/SFTP (process lifecycle, log streaming, and multi-client safety files), and EPICS CA (direct low-latency device readback, bypassing the queue-server).

ZMQ layer. The background ZMQWorker thread polls the bluesky-queueserver at approximately 1 Hz using REQ/REP messages to the control port (default 60615) and reads status documents from the info port (60625). On each poll the worker compares the new state against the previous one and emits typed Qt signals only when something changes — status_updated, console_line, devices_updated, connected, disconnected. All user-interface components are signal-driven and update reactively. A separate ZMQDocThread subscribes to the bluesky document PUB socket and forwards run documents (start, descriptor, event, stop) to the Live Viewer for real-time plotting without passing through the poll loop. Long-running calls to function_execute — used for device-status queries, PV name retrieval, and sim-device control — run in dedicated QThread worker objects so the poll loop is never blocked.

SSH layer. SSH connectivity is handled via the Paramiko library using key-pair authentication (no passwords stored). A _SSHLogTailer thread opens a persistent SSH channel and runs tail -n 50 -f <log_file> on the remote machine, streaming RE Manager stdout into the RE Console widget. The channel requests a pseudo-terminal (PTY) so that the remote sshd sends SIGHUP to the tail process if the connection closes abruptly, preventing orphaned processes. On each RunEngine Manager restart, EasyBluesky uploads the startup script (re_startup_mongo.py) and YAML permission files via SFTP before starting the new process, ensuring the remote always runs the current version without manual file transfer. Stop and start use separate SSH exec_command channels to avoid the stop channel being terminated by pkill matching its own command line.

EPICS CA layer. EPICS Channel Access monitoring is performed locally via pyepics with direct subscriptions to control-system PVs. PV names are retrieved from the live RunEngine environment once per environment open via function_execute; the resulting map is fingerprinted so that CA subscriptions are only rebuilt when the device set genuinely changes, avoiding unnecessary reconnections after a ZMQ reconnect. Callbacks from the pyepics CA thread are forwarded to the Qt main thread via queued signals and coalesced in a 100 ms flush timer, ensuring rapid PV changes produce a single tree repaint rather than per-callback redraws.

Data storage. The startup script subscribes two callbacks in the RunEngine: a suitcase.jsonl writer via a RunRouter that creates per-run JSONL files in the active experiment directory (or a local fallback), and a MongoDB writer. Both run unconditionally so that run data is preserved even when MongoDB is unavailable. A third ZMQ PUB subscription on a configurable document port feeds the Live Viewer. The application additionally maintains a lightweight plans_log.jsonl index on the client machine, recording plan name, parameters, run UID, and scan number for every completed run in the active experiment.

Thread model summary.

| Thread | Role |
| --- | --- |
| Qt main thread | All widget updates, user actions, CA signal dispatch |
| ZMQWorker (QThread) | 1 Hz poll loop; function_execute calls via sub-threads |
| ZMQDocThread (QThread) | ZMQ SUB — live bluesky documents to Live Viewer |
| _SSHLogTailer (QThread) | SSH tail -f → RE Console |
| _RunListFetcher etc. (QThread) | pymongo queries, HDF5 export, PV name fetch |
| pyepics CA thread (internal) | CA callbacks → Qt queued signals |

### 2.2 Connection Profiles

EasyBluesky stores connection parameters in a JSON file (~/.easy_bluesky/connection.json) that supports multiple named profiles. Each profile specifies the remote host, SSH key path, conda environment name, queue-server control and info ports, the devices file to load on the remote machine, and EPICS network settings (EPICS_CA_ADDR_LIST, EPICS_CA_AUTO_ADDR_LIST). Switching profiles requires a single toolbar click and reconnects automatically. No passwords are stored; authentication uses SSH key pairs only.

Profiles support both remote (SSH-managed) and local modes. A local profile starts the RunEngine Manager as a subprocess on the same machine, assigns ports automatically from a free-port range, and terminates the process cleanly on application exit. Local profiles require no SSH configuration and are the recommended starting point for new users. A simulation profile pairs a local or remote RunEngine Manager with an auto-generated devices_sim.py devices file (§3.4). A startup dialog lists all configured profiles, greying out any profile already open in another application window to enforce single-instance-per-profile operation.

### 2.3 RunEngine Manager Lifecycle

EasyBluesky provides full lifecycle control of the remote queue-server process without requiring the user to open a terminal. The restart sequence proceeds as follows: (1) an SSH channel kills the existing process via its PID file and pkill -f start-re-manager; (2) a two-second pause allows the process to exit and release ports; (3) a fresh SSH channel writes a launcher shell script to /tmp via SFTP, exporting the selected devices file path as EASY_BLUESKY_DEVICES_FILE, and starts start-re-manager wrapped in procServ for automatic restart on crash. The startup script, devices file, and a generated simulation devices file (if present) are uploaded at step (3) so that any local edits propagate to the remote automatically.

The RE console output is streamed to the interface via tail -f over SSH and displayed in a live console widget (§2.1), giving experimenters the same visibility they would have at a terminal without requiring direct shell access. The BestEffortCallback subscriber in the startup script prints a live scan table to the RE Manager log at each data point, making per-point progress visible in the console widget during running scans.

## 3. Key Features

### 3.1 Queue Management

The Queue Manager tab (Fig. 2b) provides a complete interface to the Bluesky plan queue. Experimenters can add plans by selecting from the list of plans registered in the RunEngine environment, configure plan arguments through dynamically generated forms, reorder queue items by drag-and-drop, and remove or edit items at any time. Queue execution controls — Start, Pause, Resume, Abort, and Stop — are available both in the Queue Manager and in the Experiments tab, so no navigation is required to manage a running queue.

Two automation options reduce the manual steps required during a measurement session. The Auto-start checkbox causes EasyBluesky to start the queue automatically as soon as the first plan is added, provided the RunEngine is idle and the environment is open — useful during unattended operation. The Loop control re-runs the complete queue a specified number of times (zero selects infinite looping), re-adding all items from a snapshot taken at the time the queue was started. The loop count spinbox is live: changing it during a run adjusts the number of remaining iterations immediately, and the current iteration is displayed next to the control.

### 3.2 Experiment and Data Management

EasyBluesky organises measurements into named experiments. Each experiment is a directory on the local machine that accumulates JSONL run files written by a suitcase.jsonl subscriber running inside the RunEngine environment. A plans log (plans_log.jsonl) tracks every completed plan, recording start time, plan name, parameters, and the unique run UID. The MongoDB Browser tab reads run data from a locally accessible MongoDB instance and filters the view automatically to the active experiment using run UIDs from the plans log; a regex fallback is used when the log is empty. Multiple runs can be selected simultaneously for overlay plotting, with automatic detection of common X and Y fields. Auto-plotting triggers without button presses when the selection or axis choices change.

Scan numbers are assigned at queue time rather than execution time: each plan’s md["scan_num"] is set from a counter that advances monotonically with the queue, so HDF5 filenames, MongoDB records, and the plans log are always consistent even if plans are reordered or removed before running. The current counter value is displayed as a Next scan: #N label in the Plan Log header and updates on every queue change.

A tabular data browser allows experimenters to open HDF5 archives from any completed run. Background filesystem monitoring detects if the active experiment folder is deleted or the NFS mount is lost while the app is running, pausing running scans automatically and alerting the user. RE Manager console output is appended to <exp_dir>/console.log for the duration of each session, providing a permanent record alongside the run data.

### 3.3 Live Device Monitoring

The Available Devices tab displays all devices registered in the RunEngine environment as a hierarchical tree, with columns for device class, current value, engineering units, and description (Fig. 2c). In real-beamline mode, values are updated in real time via EPICS Channel Access subscriptions using DBR_CTRL callbacks, which carry both value and engineering units in a single network message. Description fields are populated by an additional subscription to each record’s .DESC field; field suffixes (e.g. .RBV) are stripped before appending .DESC so that IOC:M1.RBV correctly fetches IOC:M1.DESC rather than the invalid IOC:M1.RBV.DESC. Callbacks from the pyepics CA thread are forwarded to the Qt main thread via queued signals and coalesced in a 100 ms timer so that rapid PV changes produce a single tree repaint rather than per-update redraws. PV names are fetched from the live RunEngine environment via function_execute on each environment open; the device fingerprint is checked before rebuilding subscriptions so that a ZMQ reconnect without a genuine environment restart does not re-subscribe unnecessarily. Units and descriptions are cached to disk (~/.easy_bluesky/device_metadata.json) after the first real-beamline session and reused in simulation mode without network access.

Motor-type devices — those exposing a user_setpoint signal — are provided with an inline tweak widget (◀ step ▶) for manual positioning without opening a separate motor screen. Mouse-wheel events on the step spinbox are suppressed to prevent accidental motor moves during scrolling.

In simulation mode (§3.4), the device tree polls the RunEngine environment at 2 Hz via read_devices_status and draws units and descriptions from the local cache. A search bar filters the tree by device name, class, or description in real time.

### 3.4 Simulation Mode

EasyBluesky includes a simulation subsystem that allows experimenters to develop and test measurement sequences on any computer, with no connection to beamline hardware. A menu action (File → Generate Sim Devices) parses the real devices file and generates a corresponding devices_sim.py that instantiates ophyd SynAxis and SynGauss surrogates for each real motor and detector. A dedicated connection profile pointing at this devices file starts a queue-server that executes plans against the synthetic devices, exercising the full Bluesky data pipeline and producing JSONL output indistinguishable in structure from real data. This capability is particularly valuable for training new users and for verifying complex multi-step sequences before committing to scarce beamtime.

### 3.5 Plan Development Environment

An integrated code editor tab (Fig. 2d) provides syntax-highlighted editing of local plan files, with the ability to add and remove plan directories that are automatically uploaded to the remote RunEngine environment. Adding a directory triggers an automatic environment reload cycle (close → upload → reopen), so newly written plans become available without manual intervention. A keyboard shortcut (Ctrl+/) toggles line comments, the editor supports standard undo/redo, and a floating Find / Replace bar (Ctrl+F / Ctrl+R) is available in all text panels including the RE Console.

### 3.6 Visual Plan Composer

The Visual Plan Composer (Fig. 2e) provides a drag-and-drop interface for assembling measurement sequences without writing Python code. Scan blocks are selected from a library that includes rel_scan, grid_scan, list_scan, count, mv, sleep, and custom energy-looping variants; each block exposes its arguments through typed form fields. Blocks can be reordered by drag-and-drop and nested to produce inner–outer motor combinations, enabling complex multi-dimensional scan patterns — for example, stepping an energy monochromator in an outer loop while running a spatial grid at each energy point — through point-and-click interaction. The Composer generates a complete Python plan definition that is displayed in the adjacent code editor and can be submitted to the queue directly, giving experienced users a starting point for further customisation.

### 3.7 ESAF Integration

At facilities that issue beam time through structured proposal and safety systems, manual transcription of proposal metadata into every experiment introduces both effort and the risk of errors. EasyBluesky integrates with Experiment Safety Assessment Form (ESAF) databases via a bundled FastAPI REST server, local PDF parsing, or manual entry (Fig. 2f). Selecting an ESAF from the picker dialog automatically creates the experiment directory in a standard hierarchy (<PI group>/<ESAF ID>_<date>/) and registers PI name, proposal title, ESAF identifier, user list, and institution as metadata fields. These fields are injected into the md (metadata) dictionary of every plan added to the queue, so they appear verbatim in run documents stored in MongoDB and in JSONL run files.

A technique dropdown in the ESAF picker narrows results to the relevant measurement modality, and a client-side regex field searches across all record fields — PI name, proposal title, ESAF ID, users, and institution — without a server round-trip. Arbitrary extra fields (custom key-value pairs) can be attached to any ESAF record and are preserved across synchronisation with the remote server. The REST server supports both MongoDB and SQLite backends, so facilities without a MongoDB installation can use a local SQLite file. Although the ESAF format used here is specific to the Advanced Photon Source, the integration pattern — form-based metadata picker feeding a standard folder hierarchy and plan metadata injection — is directly transferable to any facility with a structured proposal system.

### 3.8 Multi-client Safety

Synchrotron beamlines are frequently shared between two or more operator workstations: for example, a beamline scientist’s desktop running alongside an experimenter’s laptop, or a Windows control-room terminal and a remote Mac session connected over a VPN. The bluesky-queueserver protocol does not itself restrict the number of simultaneous ZMQ clients, so concurrent operation is technically possible but introduces the risk of conflicting commands or ambiguous state.

EasyBluesky addresses this through a multi-layered safety mechanism. An operator lock file (/tmp/.easy_bluesky_<profile>.operator) is written to the remote machine on connect and read by every subsequent client. If the file belongs to a different host, the joining client is presented with a dialog offering either to claim exclusive control (with an explicit warning that the incumbent operator is not notified) or to continue in observer mode in which RunEngine Manager start and stop are disabled. Locks older than four hours are treated as abandoned and reclaimed automatically. The lock is released on disconnect or application quit.

A connected-clients chip in the toolbar displays the live count of clients sharing the queue-server in real time, coloured green when the local machine is the sole operator and amber when multiple clients are active. Hovering over the chip shows the IP address of each connected client. The client list is determined every 30 seconds by combining two independent sources: short-lived heartbeat files written by each client to the remote machine at each poll (/tmp/.easy_bluesky_<slug>_client_<hostname>.json, expiring after 90 seconds of inactivity), and active TCP connections to the ZMQ control and info ports observed via ss -tn on the remote host. Using both sources provides resilience: heartbeat files record clients that are connected but momentarily quiet; the ss query catches clients that missed a heartbeat but still hold an open socket.

When a second client connects to an already-occupied queue-server, the first client is notified in the RE Console within one 30-second polling interval. At startup, the client count check runs before the experiment selection dialog so that the user is aware of any co-operators before choosing a measurement folder.

A further ambiguity arises when the manager_state field returned by the queue-server reports executing_task while re_state is idle. This state is entered whenever any client calls function_execute — including routine device-status polling from a second client — and is therefore not an indication that the local client’s queue is active. EasyBluesky distinguishes these cases by tracking the task name of every function_execute call it initiates (_current_task). When executing_task is observed without a matching local task, an amber BUSY (ext) chip is shown in the RE state indicator after a 1.5 s debounce, signalling external activity without alarming the operator. Local tasks are shown as BUSY: <task name> without delay. This distinction prevents the Start Queue button from being incorrectly disabled during sub-second device polls from a second client.

### 3.9 Integrated Curve Fitting

The MongoDB Browser and HDF5 Viewer tabs provide an interactive curve-fitting module built on the lmfit library (Newville et al., 2014). The fit dialog is non-modal and displays a live preview curve on the plot immediately when opened, updating in real time as parameters are adjusted. Five peak models (Gaussian, Lorentzian, Voigt, pseudo-Voigt, and split-Lorentzian), four step and interface models, and four polynomial background terms can be combined freely; six minimisation algorithms are available. Multi-peak fits are supported through a peak-count spinbox that composes an N-component model with prefixed parameters and uses a scipy.signal.find_peaks-based initial guess.

Fit parameters are remembered per dataset, keyed by the combination of run UIDs, stream name, and selected X and Y fields. Re-opening the fit dialog for a previously fitted dataset automatically restores the saved model and parameter values and re-runs the fit, restoring the full results table and curve overlay without user intervention. Switching to a different dataset opens a fresh dialog with no pre-loaded state.

First- and second-order derivatives are available directly in the browser via a Deriv dropdown using central differences (numpy.gradient), producing a result with the same number of points at the original X positions without any shift. Error bars are propagated using the exact central-difference stencil formula. Fit overlays receive the same derivative and log-Y transforms applied to the data, so they remain correctly aligned after the view is changed. Fitted parameters and curves can be exported to CSV or copied to the clipboard.

## 4. Application at the ASWAXS Beamline

EasyBluesky was developed and is in active use at the ASWAXS beamline [full beamline description]. The beamline uses a Pilatus 300K area detector (Dectris Ltd) controlled through an ophyd EpicsAreaDetector device. [Add 2–3 sentences describing a representative experiment — e.g. a SAXS or WAXS scan, number of plans in the queue, data volume — and include the scan data figure here.]

Fig. 3 shows a representative motor scan acquired through EasyBluesky. [Describe the scan: sample, motor, range, step size, signal, and what the peak represents.] The scan was defined and submitted to the queue entirely through the graphical interface without user interaction with a Python interpreter. The RunEngine Manager ran on a dedicated Linux workstation and was controlled from a MacBook Pro over the facility network; latency between queue operations and RunEngine response was consistently below 200 ms.

## 5. Conclusions

EasyBluesky provides a complete, self-contained graphical interface to the Bluesky queue-server that requires no Python knowledge to operate. By encapsulating SSH connectivity, RunEngine lifecycle management, EPICS device monitoring, experiment logging, Visual Plan Composer, and plan development in a single desktop application, it substantially lowers the barrier to autonomous operation of Bluesky-controlled beamlines. The simulation mode enables beamtime preparation on personal hardware and supports user training independent of beamline availability.

Beyond single-user operation, EasyBluesky addresses the practical reality of shared beamline infrastructure through its multi-client safety layer. The combination of operator locking, live client-count display, and BUSY-state disambiguation makes concurrent ZMQ connections safe and transparent in a way that the queue-server protocol alone does not provide. ESAF integration reduces the manual overhead of experiment bookkeeping and ensures that proposal metadata is captured consistently in every run document. The integrated curve fitting module with per-dataset memory closes the analysis loop within the acquisition interface, reducing the time between scan completion and quantitative result.

EasyBluesky is open source, actively maintained, and designed to be deployable at any facility running bluesky-queueserver 0.0.25 or later. Although developed at the ASWAXS beamline at NSF’s ChemMatCARS (Sector 15, Advanced Photon Source), the application makes no assumptions about the underlying control system beyond the presence of a bluesky-queueserver ZMQ interface, and has been tested against both EPICS-based real beamlines and fully simulated environments. Source code, installation instructions, and documentation are available at https://github.com/nayanbera/easy-bluesky.

## Acknowledgements

[Funding sources, beamline staff contributions, facility acknowledgement.]

## Figure Captions

Figure 1. Architecture of the EasyBluesky client–server system. The desktop application runs on the experimenter’s local workstation (Mac or PC) and communicates with the bluesky-queueserver running on a dedicated beamline Linux computer via three independent channels. (i) ZMQ REQ/REP sockets (control port 60615, info port 60625) are used by the ZMQWorker poll thread for queue operations and 1 Hz status polling; a separate ZMQ SUB socket on the document port feeds the Live Viewer with run documents in real time. (ii) SSH (Paramiko, key-pair only) is used by a persistent _SSHLogTailer thread to stream the RE Manager log via tail -f, and by explicit commands for lifecycle operations (start, stop, restart) and SFTP upload of the startup script, permissions file, and devices file. (iii) EPICS Channel Access (CA) subscriptions are established directly from the local machine to the beamline control system, bypassing the queue-server entirely for low-latency, sub-millisecond device readback. PV names are fetched once per environment open via function_execute. For shared-beamline deployments, an operator lock file and client heartbeat files are maintained on the remote machine via the SSH channel; active TCP connections to the ZMQ ports are additionally queried via ss -tn to provide a second, independent view of the connected-client count. All blocking operations (pymongo queries, HDF5 export, PV name retrieval, SFTP upload) run in QThread worker objects; the Qt main thread handles only widget updates and user actions.

Figure 2. EasyBluesky graphical interface. (a) Main application window showing the tab-based layout: Experiments, Queue Manager, Available Devices & Plans, Code Editor, RE Console, and Data Browser tabs are accessible from the top navigation bar. The persistent RE control bar at the top displays the current RunEngine state chip (IDLE, RUNNING, PAUSED, BUSY, or BUSY (ext)), environment state, and the connected-clients chip (green for sole operator, amber when sharing). (b) Queue Manager tab showing the plan queue (left panel) with drag-and-drop reordering, queue execution controls (Start, Pause, Resume, Abort, Stop), Auto-start checkbox, and Loop controls with an iteration counter. Each queued plan displays its pre-assigned scan number (#N). The right panel shows the plan detail view and RE console output. (c) Available Devices tab displaying the live device tree with columns for device class, current value, engineering units, and EPICS description. Motor-type devices expose inline tweak controls (◀ step ▶) for manual positioning. The search bar above the tree filters entries by name, class, or description. (d) Code Editor tab with syntax-highlighted Python plan editing, Plan Files management (add/remove local plan directories), and the integrated plan builder for constructing queue items from registered plans without writing code. (e) Visual Plan Composer showing drag-and-drop scan blocks arranged into a multi-step sequence with inner and outer motor loops; the right panel shows the generated Python code. (f) ESAF picker dialog showing the technique filter dropdown, client-side regex search field, and the selected ESAF record with PI name, proposal title, and user list; clicking Launch creates the experiment folder and registers the metadata.

Figure 3. Representative motor scan acquired using EasyBluesky at the ASWAXS beamline. [Motor name] was stepped through [range and units] in [N] steps; the signal recorded at each point was [detector/channel]. The peak at [position and units] corresponds to [physical meaning — e.g. the rocking curve maximum, the knife-edge midpoint]. The scan was defined and submitted to the queue entirely through the EasyBluesky graphical interface without user interaction with a Python interpreter. Data were written automatically to a JSONL run file and are shown here as plotted by [your plotting tool — e.g. the integrated data browser / matplotlib].

## References

Allan, D., Caswell, T., Campbell, S. & Rakitin, M. (2019). Bluesky’s ahead: A multi-facility collaboration for an a la carte software project for data acquisition and management. Synchrotron Radiation News, 32(3), 19–22.

Brookhaven National Laboratory (2014). ophyd: Python hardware abstraction library for the Bluesky ecosystem. https://github.com/bluesky/ophyd

Caswell, T. A., Allan, D., Campbell, S., Lauer, K., Rakitin, M., Sauter, N. & Sutton, M. (2021). bluesky-queueserver: A queue for bluesky plans. https://github.com/bluesky/bluesky-queueserver

Chin, L. & Leiserson, C. E. (2022). PyZMQ: Python bindings for ZeroMQ. https://github.com/zeromq/pyzmq

Engström, J. (2020). Paramiko: Python SSH2 library. https://github.com/paramiko/paramiko

Newville, M., Stensitzki, T., Allen, D. B. & Ingargiola, A. (2014). LMFIT: Non-linear least-square minimization and curve-fitting for Python. Zenodo. https://doi.org/10.5281/zenodo.11813

Rivers, M. (2012). pyepics: Python interface to EPICS Channel Access. Proceedings of the 14th International Conference on Accelerator and Large Experimental Physics Control Systems (ICALEPCS 2013), San Francisco, CA.

The Qt Company (2022). Qt Framework (version 6). https://www.qt.io

[Add suitcase.jsonl / event-model reference if published] [Add MongoDB reference if needed by journal style]